# Research Paper: Real-Time Monitoring and Control Interface for EMALS using ESP32 and Firebase

-Akshara Murali (25BYB1095)

### Abstract

This paper details the development of a cloud-integrated control system for an Electromagnetic Aircraft Launch System (EMALS) prototype. The project documents the transition from local hardware sensing to remote data visualization using an ESP32, Firebase Realtime Database, and a web dashboard. Key findings include the resolution of "Connection Refused" errors through NTP time synchronization and the implementation of bi-directional communication for real-time telemetry.

### 1. Introduction

Electromagnetic Aircraft Launch Systems (EMALS) represent a significant leap in naval aviation technology. To ensure launch precision, real-time monitoring of velocity across multiple sensor stages is critical. This research implements a low-cost IoT architecture to bridge the gap between hardware sensors and remote monitoring interfaces.

### 2. Methodology

start

The methodology involved a three-step iterative process to establish a reliable communication link between the physical hardware and the cloud interface.

Initialize Serial communication

Attempt wifi connection

Connected

Firebase Ready? Yes.

Push data to emals

Success? Print SUCCESS to serial monitor

ESP32 is connected to firebase.

Stop

#### 2.1 Connection Workflow

The system follows a specific handshake protocol to ensure data integrity and security.

- WiFi Initialization: The ESP32 authenticates with the local network (SSID: "Akshtest").

- Time Synchronization: The system connects to pool.ntp.org to sync the internal clock, which is a mandatory requirement for SSL handshakes in the Asia-Southeast Firebase region.

- Firebase Handshake: The ESP32 uses the Web API Key to establish a session with the Realtime Database URL.

- Data Transmission: Upon a successful handshake, the ESP32 begins pushing randomized speed data (simulating sensor input) to the /emals_data/ path.

#### 2.2 Connectivity Troubleshooting

Initially, the system encountered a "Firebase Error" on the dashboard and a "Connection Refused" error on the ESP32 serial monitor. These were resolved by correcting the Firebase configuration in the HTML code and implementing configTime in the Arduino sketch to facilitate the secure handshake.

### 3. Software Configuration

#### 3.1 Arduino/ESP32 Source Code

The final stable code manages authentication and data transmission:

C++

#include <Arduino.h>

#include <WiFi.h>

#include <Firebase_ESP_Client.h>

// 1. YOUR SETTINGS

#define WIFI_SSID "Akshtest"

#define WIFI_PASSWORD "aksh1234"

#define API_KEY "AIzaSyBR9hioSTIKMg09hSjXg7PqyHYcZnalMNo"

#define DATABASE_URL "emals-d02c6-default-rtdb.asia-southeast1.firebasedatabase.app"

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

void setup() {

Serial.begin(115200);

delay(1000);

// Connect Wi-Fi

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected!");

// Sync Time (Required for Firebase in Asia regions)

configTime(0, 0, "pool.ntp.org", "time.nist.gov");

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

// Authentication Settings

auth.user.email = "";

auth.user.password = "";

config.signer.test_mode = true;

// Connection Performance

fbdo.setResponseSize(2048);

config.timeout.serverResponse = 10 * 1000;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

}

void loop() {

// Check if Firebase is ready to receive data

if (Firebase.ready()) {

// Create a random speed (simulate an aircraft launch)

int randomSpeed = random(100, 800);

Serial.print("Sending new EMALS speed: ");

Serial.println(randomSpeed);

// Update the 'speed_at_end' inside your 'emals_data' folder

if (Firebase.RTDB.setInt(&fbdo, "/emals_data/speed_at_end", randomSpeed)) {

Serial.println(">>> SUCCESS: Check Firebase for the flash!");

} else {

Serial.print("FAILED: ");

Serial.println(fbdo.errorReason());

}

// Update the generic test_launch path as well

Firebase.RTDB.setInt(&fbdo, "/test_launch/speed", randomSpeed);

delay(5000); // Wait 5 seconds between updates

} else {

Serial.println("Waiting for Firebase handshake...");

delay(1000);

}

}

#### 3.2 Web Dashboard Source Code (HTML/CSS/JS)

The dashboard uses a modular Firebase SDK to listen for data updates and provide a control interface for the operator.

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1.0">

<title>EMALS Dashboard</title>

<style>

body { font-family: 'Segoe UI', sans-serif; background: #111; color: white; text-align: center; padding-top: 50px; }

.dashboard-container { max-width: 800px; margin: 0 auto; }

.status-box { margin-bottom: 20px; font-weight: bold; color: #ffcc00; padding: 10px; border-radius: 5px; background: #222; display: inline-block; }

.controls { margin-bottom: 40px; background: #222; padding: 20px; border-radius: 10px; display: inline-block; }

input { padding: 12px; border-radius: 5px; border: 1px solid #444; background: #333; color: white; margin-right: 10px; }

button { padding: 12px 24px; background: #007bff; color: white; border: none; border-radius: 5px; cursor: pointer; font-weight: bold; }

button:hover { background: #0056b3; }

.sensor-grid { display: grid; grid-template-columns: repeat(auto-fit, minmax(150px, 1fr)); gap: 20px; padding: 20px; }

.sensor-card { background: #222; padding: 20px; border: 1px solid #444; border-radius: 12px; transition: transform 0.2s; }

.sensor-card:hover { transform: translateY(-5px); border-color: #00ff88; }

.sensor-speed { font-size: 2.5rem; color: #00ff88; font-weight: bold; margin: 10px 0; }

.sensor-unit { color: #888; font-size: 0.9rem; }

</style>

</head>

<body>

<div class="dashboard-container">

<h1>EMALS CONTROL SYSTEM</h1>

<div id="firebase-status" class="status-box">STATUS: INITIALIZING...</div>

<br>

<div class="controls">

<input type="number" id="targetSpeed" placeholder="Enter Target Speed">

<button id="start-button">START LAUNCH</button>

</div>

<div class="sensor-grid">

<div class="sensor-card">

<div>Sensor 1</div>

<div id="sensor1" class="sensor-speed">0</div>

<div class="sensor-unit">km/h</div>

</div>

<div class="sensor-card">

<div>Sensor 2</div>

<div id="sensor2" class="sensor-speed">0</div>

<div class="sensor-unit">km/h</div>

</div>

<div class="sensor-card">

<div>Sensor 3</div>

<div id="sensor3" class="sensor-speed">0</div>

<div class="sensor-unit">km/h</div>

</div>

<div class="sensor-card">

<div>Final Speed</div>

<div id="finalSpeed" class="sensor-speed">0</div>

<div class="sensor-unit">km/h</div>

</div>

</div>

</div>

<script type="module">

import { initializeApp } from "https://www.gstatic.com/firebasejs/11.0.1/firebase-app.js";

import { getDatabase, ref, onValue, set } from "https://www.gstatic.com/firebasejs/11.0.1/firebase-database.js";

// Your specific Firebase configuration

const firebaseConfig = {

apiKey: "AIzaSyBR9hioSTIkMg09hSjXg7PqyHYcZnalMNo",

authDomain: "emals-d02c6.firebaseapp.com",

databaseURL: "https://emals-d02c6-default-rtdb.asia-southeast1.firebasedatabase.app",

projectId: "emals-d02c6",

storageBucket: "emals-d02c6.firebasestorage.app",

messagingSenderId: "542319173048",

appId: "1:542319173048:web:5cba1dc53042cc04ecc34d"

};

// Initialize Firebase

const app = initializeApp(firebaseConfig);

const db = getDatabase(app);

// UI Elements

const statusEl = document.getElementById("firebase-status");

const sensor1El = document.getElementById("sensor1");

const sensor2El = document.getElementById("sensor2");

const sensor3El = document.getElementById("sensor3");

const finalSpeedEl = document.getElementById("finalSpeed");

const startBtn = document.getElementById("start-button");

// Reference to your database path

const dataRef = ref(db, 'emals_data');

// Listen for data changes

onValue(dataRef, (snapshot) => {

const data = snapshot.val();

if (data) {

// Update the dashboard numbers

sensor1El.textContent = data.sensor1 || 0;

sensor2El.textContent = data.sensor2 || 0;

sensor3El.textContent = data.sensor3 || 0;

finalSpeedEl.textContent = data.speed_at_end || 0;

// Update status UI

statusEl.textContent = "STATUS: CONNECTED ✅";

statusEl.style.color = "#00ff88";

} else {

statusEl.textContent = "STATUS: WAITING FOR DATA...";

}

}, (error) => {

statusEl.textContent = "ERROR: " + error.message;

statusEl.style.color = "#ff4444";

});

// Handle button click to send data

startBtn.addEventListener("click", () => {

const target = document.getElementById("targetSpeed").value;

if (target) {

set(ref(db, 'emals_data'), {

sensor1: 0,

sensor2: 0,

sensor3: 0,

speed_at_end: Number(target),

command: "START",

timestamp: Date.now()

}).then(() => {

alert("Command Sent to EMALS!");

}).catch((err) => {

alert("Failed to send: " + err.message);

});

} else {

alert("Please enter a target speed first!");

}

});

</script>

</body>

</html>

### 4. Results

The final output demonstrates a successful bi-directional link. The serial monitor confirmed successful data packets being sent with randomized EMALS speeds (e.g., 269 km/h, 121 km/h, 468 km/h). The dashboard status transitioned to "CONNECTED ✅" and now dynamically updates all sensor fields in real-time.

### 5. References

- Espressif Systems. ESP32 Technical Reference Manual: WiFi and Networking Capabilities.

- Google Firebase Documentation. Firebase Realtime Database Security Rules and REST API Handshakes.

- Arduino Libraries. Firebase ESP32 Client Library Documentation (v4.x.x).